





SCHEDULE OF WORK   -nit-37
Name of work:- D/O Spritual park at Kalkaji adjacent at Nehru Place( Revised name as Aastha Kunj) (SH: Providing 


       path lights near gate at Kailash Hills and Shambhu Dayal park).

	S. No.
	Description of item
	Qty
	Unit
	Rate 
	Amt.

	1.
	Supply ,erection, testing  and commissioning of GI pole 65mm dia ‘B” class of 6 mtr. height duly welded with M.S base plate of size  (200X200X6mm), M.S. connector box welded on the pole duly painted with following accessories thereon, G.I. pipe sleeve 75 mm dia for cable entry /clamp for fittings as per and details below erection in cement, concrete foundation 1:3:6(1 cement, 3 coarse sand 6 graded stone 40 mm nominal size /excavation re-fitting etc. as reqd.

a) 6A SP  MCB-1No.

b) 16A connecation-2Nos.

c) Copper  busbar-1 set.
	25 Nos.
	Each
	
	

	2.
	Supply, installation and commissioning of street light luminaires on M.S. brackets as shown in drawing for 2X18 W  CFL complete with2 Nos. 16 watt CFL lamps ,for  operation  on 200 to 240 volt 50HZ A.C. supply complete with control gear, reflector, protective glass cover complete as per technical specification etc. as reqd.


	25 Nos.
	Each
	
	

	3.
	Supplying of 2X16 sqmm Al. conductor XLPE insulated PVC sheathed armoured power cable of 1.1 KV grade as reqd.


	1185 mtr.
	p/mtr.
	
	

	4.
	Supplying & drawing of following sizes of PVC insulated copper conductor single core cable in the existing surface/recess steel conduit /GI pipe etc. as reqd.( for connection between cable connector & fitting .

a) 3X1.5 sqmm.
	150 mtr.
	p/mtr.
	
	

	5.
	Laying one number PVC insulated & PVC sheathed XLPE power cable of 1.1 KV grade of size not exceeding25 sqmm i/c excavation and  sand cushioning, protective covering & refilling the trench etc. as reqd.
	685 mtr.
	p/mtr.
	
	

	6.
	Laying of one number additional PVC insulated PVC sheathed XLPE power cable of 1.1 kV grade of size not exceeding 25 sqmm direct in ground in the same trench in one tier horizontal formation/i/c excavation sand cushioning protective & re-filling the trench etc. as reqd.
	500 mtr.
	p/mtr.
	
	

	7.
	Supplying & fixing following way single pole and natural sheet steel MCB distribution board 240 volts on surface/recess complete with tinned copper bus bar, netural bus bar, earth bar, din bar, detachable gland plate, meter connection ,phosphate and powder painted including ear thing etc. as reqd.(without MCB/RCCB isolator)

a) 8 way single door.
	1 No
	Each
	
	

	
	
	
	
	
	


	8.
	Supplying and fixing 5 amp to 32 amps, rating 240 volt ‘B’ series miniature circuit breaker suitable for lighting & other loads of following pole with existing MCB DB complete with connection, testing and commissioning etc. as reqd

i) Single pole 10 Amp.

ii) Triple pole and natural.

iii) Blanking plate.
	3 Nos.

1 No.

1No.
	Each

-do-

-do-
	
	

	9.
	G.I. earth pipe 4.5 mtrs. Long 40 mm dia i/c accessories and providing masonary enclosure with cover plate having locking arrangement and watering pipe etc. with charcoal and salt as reqd.
	4 set.
	p/set
	
	

	10.
	Providing & fixing 6 SWG dia G.I. wire with cable as reqd.
	1285 Sq.mtr.
	p/sqmtr.
	
	

	
	
	
	
	
	


ADDITIONAL CONDITION:-nit-37
1.
The work shall be carried out as per CPWD Specification and terms & 
conditions of the agreement & to the entire satisfaction of Engineer-in-
charge.

2.
The contractor must get himself acquainted with the proposed site of work & 
study specification & conditions carefully before tendering .If site is not 
available for any reason, the program me of execution shall be modified 
accordingly & contactor shall have no claim except providing the print 
conditions of the contract.

3.
CPWD General specification for electrical works part I&II internal & external 
2007with up to date amendment, and relevant IS: Specifications.

4.
Related Indian standard code of practice.

5.
The rates quoted by the contractor shall be firm inclusive of all central/state 
Govt. taxes, duties, levies nothing extra shall be paid on this account.
6.
Concessional sale tax for C/D shall not be issued by the department.

7.
No T&P shall be provided by the department.

8.
Contractor shall make its own arrangement of water, electricity reqd for 
execution.

9.
The contractor shall got approved material from Engineer-in-charge before 
use at site.

10.
All material to be used by the contractor shall be new.

11.
The fitting shall be of Crompton/Surya/Bajaj make..

12.
G.I. pipe pole shall be of ISI make Jindal /TaTa


The contactor shall submit completion certificate and completion plan.


The completion plan shall be submitted as under.

i)
On blue print-3 Nos.

ii)
On Tracing cloth-1No.

13.
Work executed by the contractor shall be guarantee by the contactor for one 
year for smooth for functioning and against manufacturing defecate. 
Defective material if found any shall be replaced by the contractor of cost. 

14.
The cable shall be tested asunder :


i) Before laying (ii) After laying (iii) after back fitting & running & shall be 
recorded in insulation test resister & shall be submitted to deptt

15.    Earth test shall be recorded in earth test register & submitted to the deptt.

TECHNICAL SPECIFICATION:-nit-37
1.
The G.I pipe of 65mm dia shall be of ‘B’ class medium size of TATA,JINDAL & 
Jindal ,Hissar make.

2.
The light fittings of Crompton Groves/Surya Bajaj  make.

3.
the XLPE insulated PVC sheathed cable aluminium cable shall be of Sky 
tone/Grandly make.

4.
MCB,TPN switch & MCCB box of STANDARD/INDDO COPPE/HAVELL’S.

5.
Copper conductor wire from connector box to luminaries of single & 
conductor PVC insulated Grandlay /Surya/ Sky tone make.

            DELHI DEVELOPMENT AUTAHORITY




    ELECTRICAL DIVISON NO.11



                VIKAS SADAN: INA: NEW DELHI: 23.

NIT NO.37/EE/El. DivisionNo.11/DDA/09-10

NOTICE INVITING TENDER
Sealed item rate tender are invited for the following works on behalf of DDA from Firms enlisted/approved for similar nature of works in appropriate class of DDA,CPWD,MES & P & T and Railway which shall be opened on the same day at 3.30 P:M at Ground Floor, Vikas Minar IP Estate, New Delhi by the under signed in the presence of intending purchaser or their authorized  representatives. 

Tender documents can be had on any working day between 10.00 AM to 3.00 PM on payment of the amount mentioned against the work (non refundable) on production of valid Enlistment Registration Certificate and valid Registration with Works Contract Cell of Sales Tax Deptt. of NTC and Tax Clearance Certificate issued by Works Contract Cell in Form XI  at  Sr.AO(South Zone ),Shahpur Jat and Centralized tender office at Vikas Minar, IP Estate, New Delhi.  Earnest Money shall be submitted along with tender documents in the form of CDR/Demand Draft of Scheduled Bank Guaranteed by the Reserve Bank of India in favour of  Sr. Accounts Officer/CAU(South Zone) DDA. The tender of the contractor who does not enclose the earnest money on prescribed manner shall be summarily rejected.

Tender can be submitted on specified date upto 3.00 PM which shall be opened on the same day at 3.30 PM at Ground Floor, Vikas Minar IP Estate, New Delhi by the undersigned in the presence of the contractors /theirs authorized representatives .

___________________________________________________________________________________
S.No .       
 Name of Works


 Estt.cost        
       Cost of Tender      Last dt. of sale 





    
Earnest Money         Time allowed       Dt. of opening  
____________________________________________________________________________________
  1.   D/O Spiritual park at Kalkaji adjacent Nehru 

 Place(Revised name as  Aastha Kunj SH;

Rs.4,30,873/-
         Rs.500/-+4 % Vat    11/12/09
 Providing path lights  near gate at Kalish Hills &  Rs.8,617/-                     One Month            15/12/09

  Shambhu Dayal Park . 

NIT No.37/EE/ED-11/09-10

Sd.02-12-2009

Executive Engineer(E)

Electrical  Division No.11/DDA










C0ntd.2/-







2/-

(Misc.)/EE/El.Div. No.11/DDA/09-10________                         dated________
Copy to:-

1. The Chief Engineer(Elect.), DDA,

2. The Suptdg. Engineer (Elect.) Circle-3, DDA.

3. The Suptdg, Engineer/Quality Control Cell, DDA.

4. Director System, DDA, with the request to put on DDA “Websit.

5. Finance Officer, to Chief Engineer9Elect.0, DDA

6. Executive Engineer/Western Division No. 5,Vikas Minar, DDA

7. Executive Engineer 9Elect.) Div. No. 1,3,4,5,6,78,9,10 &12/DDA.

8. Sr.Accounts Officer, CAU/South Zone /DDA.

9. Asstt. Engineer (Elect.)1,2,3,4 & AE (P)Dv. No.11/DDA.

10. Asstt. Accounts Officer, Elect. Div. No. 11 .DDA.

11. Head Clerk, Elect. Div. No.11 /DDA.

12. The Association of Electrical Contractor (Regd.) Chamber No.103,Laxmi Complex, 

B-6, Subhash Chowk, Laxmi Nagar, Delhi

13. DDA Contractor Association,. E-18, Vikas Kutteer New Delhi.

14. Secretary, Contractor Association. Block-11, 946, Kirti Nagar, Ware Housing Scheme, New Delhi.

15. DDA Contractor Welfare Association. Room No. 12, Barrack No.1, Vikas Kutteer, New Delhi.

16. N.I.T. Copy.

17. Notice Board.
Sd.02-12-2009
 EXECUTIVE ENGINER (E)
 ELECT.DIVN.NO.11/DDA

   NEW DELHI.

